Listingi do projektu „Sterownik graficznego wyświetlacza LCD 8,4” typu EG9018C

dla mikrokontrolera STM32 - Aleksander Borysiuk

Listing 1: Funkcja konfigurująca układy mikrokontrolera STM32 obsługujące wyświetlacz EG9018C

void EG9018C_Init(void)

{

GPIO_InitTypeDef GPIO_InitStructure;

TIM_TimeBaseInitTypeDef TIM_TimeBaseStructure;

TIM_OCInitTypeDef TIM_OCInitStructure;

TIM_BDTRInitTypeDef TIM_BDTRInitStructure;

DMA_InitTypeDef DMA_InitStructure;

/// --------------------------------------------------------------------------

///                      Clock enable of used peripherals

///         EG9018C uses DMA1, TIM1, TIM4, TIM5, GPIOA, GPIOD and AFIO

/// --------------------------------------------------------------------------

RCC_AHBPeriphClockCmd(RCC_AHBPeriph_DMA1, ENABLE);

RCC_APB1PeriphClockCmd(RCC_APB1Periph_TIM4 | RCC_APB1Periph_TIM5, ENABLE);

RCC_APB2PeriphClockCmd(RCC_APB2Periph_GPIOA | RCC_APB2Periph_GPIOD | \

RCC_APB2Periph_AFIO | RCC_APB2Periph_TIM1, ENABLE);

/// --------------------------------------------------------------------------

///                Configuration of LCD data lines

///       EG9018C data lines are connected to pins PD7..PD0

/// --------------------------------------------------------------------------

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_7 | GPIO_Pin_6 | GPIO_Pin_5 | GPIO_Pin_4 |\

GPIO_Pin_3 | GPIO_Pin_2 | GPIO_Pin_1 | GPIO_Pin_0;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_Out_PP;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_Init(EG9018C_DATA, &GPIO_InitStructure);

/// --------------------------------------------------------------------------

///           Configuration of LCD data clock CP2 and BACKLIGHT on/off

/// EG9018C data clock CP2 is generated on pin PD12 by channel 1 of timer TIM4

/// Backlight on/off sigal is generated on pin PD13 by channel 2 of timer TIM4

/// --------------------------------------------------------------------------

// PD12 configuration as TIM4_CH1 output, PD13 configuration as TIM4_CH2 output

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_12 | GPIO_Pin_13;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_AF_PP;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_Init(GPIOD, &GPIO_InitStructure);

GPIO_PinRemapConfig(GPIO_Remap_TIM4, ENABLE);

// TIM4 time base configuration (f_clk=72MHz)

TIM_TimeBaseStructure.TIM_Period = EG9018C_CP2_PERIOD-1;

TIM_TimeBaseStructure.TIM_Prescaler = 0;

TIM_TimeBaseStructure.TIM_ClockDivision = 0;

TIM_TimeBaseStructure.TIM_CounterMode = TIM_CounterMode_Up;

TIM_TimeBaseInit(TIM4, &TIM_TimeBaseStructure);

TIM_ARRPreloadConfig(TIM4, ENABLE);

// TIM4 Channel1 configuration: PWM1 Mode / 50%

TIM_OCInitStructure.TIM_OCMode = TIM_OCMode_PWM1;

TIM_OCInitStructure.TIM_Pulse = EG9018C_CP2_PERIOD/2;

TIM_OCInitStructure.TIM_OutputState = TIM_OutputState_Enable;

TIM_OCInitStructure.TIM_OCPolarity = TIM_OCPolarity_High;

TIM_OC1Init(TIM4, &TIM_OCInitStructure);

TIM_OC1PreloadConfig(TIM4, TIM_OCPreload_Enable);

// TIM4 Channel2 configuration: PWM1 Mode / 0% or 100%

TIM_OCInitStructure.TIM_OCMode = TIM_OCMode_PWM1;

TIM_OCInitStructure.TIM_Pulse = EG9018C_BKLT_OFF;

TIM_OCInitStructure.TIM_OutputState = TIM_OutputState_Enable;

TIM_OCInitStructure.TIM_OCPolarity = TIM_OCPolarity_High;

TIM_OC2Init(TIM4, &TIM_OCInitStructure);

TIM_OC2PreloadConfig(TIM4, TIM_OCPreload_Enable);

// Clock source configuration: TIM4 is the master for timers TIM2 and TIM5

TIM_SelectMasterSlaveMode(TIM4, TIM_MasterSlaveMode_Enable);

TIM_SelectOutputTrigger(TIM4, TIM_TRGOSource_Update);

/// --------------------------------------------------------------------------

///        Configuration of LCD data latch CP1 and CONTRAST adjustment

/// EG9018C data latch CP1 is generated on pin PA0 by channel 1 of timer TIM5

/// Contrast adjustment voltage is generated on pin PA1 by channel 2 of timer TIM5

/// --------------------------------------------------------------------------

// PA0 configuration as TIM5_CH1 output, PA1 configuration as TIM5_CH2 output

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_0 | GPIO_Pin_1;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_AF_PP;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_Init(GPIOA, &GPIO_InitStructure);

// Clock source configuration: TIM5 is the slave of TIM4

TIM_SelectSlaveMode(TIM5, TIM_SlaveMode_External1);

TIM_SelectInputTrigger(TIM5, TIM_TS_ITR2);

// TIM5 time base configuration (f_clk=fTIM4)

TIM_TimeBaseStructure.TIM_Period = EG9018C_CP1_PERIOD-1;

TIM_TimeBaseStructure.TIM_Prescaler = 0;

TIM_TimeBaseStructure.TIM_ClockDivision = 0;

TIM_TimeBaseStructure.TIM_CounterMode = TIM_CounterMode_Up;

TIM_TimeBaseInit(TIM5, &TIM_TimeBaseStructure);

TIM_ARRPreloadConfig(TIM5, ENABLE);

// TIM5 Channel1 configuration: PWM1 Mode

TIM_OCInitStructure.TIM_OCMode = TIM_OCMode_PWM1;

TIM_OCInitStructure.TIM_Pulse = EG9018C_CP1_PERIOD-EG9018C_CP1_LEN;

TIM_OCInitStructure.TIM_OutputState = TIM_OutputState_Enable;

TIM_OCInitStructure.TIM_OCPolarity = TIM_OCPolarity_Low;

TIM_OC1Init(TIM5, &TIM_OCInitStructure);

TIM_OC1PreloadConfig(TIM5, TIM_OCPreload_Enable);

// TIM5 Channel2 configuration: PWM1 Mode

TIM_OCInitStructure.TIM_OCMode = TIM_OCMode_PWM1;

TIM_OCInitStructure.TIM_Pulse = EG9018C_CNTR_ADJ;

TIM_OCInitStructure.TIM_OutputState = TIM_OutputState_Enable;

TIM_OCInitStructure.TIM_OCPolarity = TIM_OCPolarity_High;

TIM_OC2Init(TIM5, &TIM_OCInitStructure);

TIM_OC2PreloadConfig(TIM5, TIM_OCPreload_Enable);

/// --------------------------------------------------------------------------

///                  Configuration of LCD start-up S

/// EG9018C start-up S is generated on pin PA8 by channel 1 of timer TIM1

/// Start-up pulse delay is provided by TIM1 OCx output dead time generator

/// --------------------------------------------------------------------------

// PA8 configuration as TIM1_CH1 output

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_8;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_AF_PP;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_Init(GPIOA, &GPIO_InitStructure);

// Clock source configuration: TIM1 is the slave of TIM4

TIM_SelectSlaveMode(TIM1, TIM_SlaveMode_External1);

TIM_SelectInputTrigger(TIM1, TIM_TS_ITR3);

// TIM1 time base configuration (f_clk=fTIM4)

TIM_TimeBaseStructure.TIM_Period = EG9018C_S_PERIOD-1;

TIM_TimeBaseStructure.TIM_Prescaler = 0;

TIM_TimeBaseStructure.TIM_ClockDivision = 0;

TIM_TimeBaseStructure.TIM_CounterMode = TIM_CounterMode_Up;

TIM_TimeBaseStructure.TIM_RepetitionCounter = 0;

TIM_TimeBaseInit(TIM1, &TIM_TimeBaseStructure);

TIM_ARRPreloadConfig(TIM1, ENABLE);

// TIM1 Channel1 configuration: PWM1 Mode

TIM_OCInitStructure.TIM_OCMode = TIM_OCMode_PWM1;

TIM_OCInitStructure.TIM_Pulse = EG9018C_S_LEN;

TIM_OCInitStructure.TIM_OutputState = TIM_OutputState_Enable;

TIM_OCInitStructure.TIM_OCPolarity = TIM_OCPolarity_High;

TIM_OCInitStructure.TIM_OCIdleState = TIM_OCIdleState_Reset;

TIM_OCInitStructure.TIM_OutputNState = TIM_OutputNState_Enable;

TIM_OCInitStructure.TIM_OCNPolarity = TIM_OCNPolarity_High;

TIM_OCInitStructure.TIM_OCNIdleState = TIM_OCNIdleState_Reset;

TIM_OC1Init(TIM1, &TIM_OCInitStructure);

TIM_OC1PreloadConfig(TIM1, TIM_OCPreload_Enable);

// TIM1 Dead Time

TIM_BDTRInitStructure.TIM_DeadTime = 0xFF;

TIM_BDTRInitStructure.TIM_AutomaticOutput = TIM_AutomaticOutput_Disable;

TIM_BDTRInitStructure.TIM_Break = TIM_Break_Disable;

TIM_BDTRInitStructure.TIM_BreakPolarity = TIM_BreakPolarity_Low;

TIM_BDTRInitStructure.TIM_LOCKLevel = TIM_LOCKLevel_OFF;

TIM_BDTRInitStructure.TIM_OSSIState = TIM_OSSIState_Disable;

TIM_BDTRInitStructure.TIM_OSSRState = TIM_OSSRState_Disable;

TIM_BDTRConfig (TIM1,&TIM_BDTRInitStructure);

// TIM1 Main Outputs Enable

TIM_CtrlPWMOutputs(TIM1, ENABLE);

/// --------------------------------------------------------------------------

///                  Configuration of LCD data transfer

/// EG9018C data is transfered to pins PD7..PD0 by DMA1 controller via channel 1

/// DMA transfer is requested by compare event in channel 1 of timer TIM4

/// --------------------------------------------------------------------------

// DMA1 Channel 1 configuration

DMA_DeInit(DMA1_Channel1);

DMA_InitStructure.DMA_PeripheralBaseAddr = (uint32_t)&(EG9018C_DATA->ODR);

DMA_InitStructure.DMA_MemoryBaseAddr = (uint32_t)&EG9018C_frame_buffer[0][0];

DMA_InitStructure.DMA_DIR = DMA_DIR_PeripheralDST;

DMA_InitStructure.DMA_BufferSize = (EG9018C_XRES/4)*(EG9018C_YRES/2);

DMA_InitStructure.DMA_PeripheralInc = DMA_PeripheralInc_Disable;

DMA_InitStructure.DMA_MemoryInc = DMA_MemoryInc_Enable;

DMA_InitStructure.DMA_PeripheralDataSize = DMA_PeripheralDataSize_Word;

DMA_InitStructure.DMA_MemoryDataSize = DMA_MemoryDataSize_Byte;

DMA_InitStructure.DMA_Mode = DMA_Mode_Circular;

DMA_InitStructure.DMA_Priority = DMA_Priority_VeryHigh;

DMA_InitStructure.DMA_M2M = DMA_M2M_Disable;

DMA_Init(DMA1_Channel1, &DMA_InitStructure);

// Enable DMA1 Channel 1

DMA_Cmd(DMA1_Channel1, ENABLE);

// Set compare event in channel 1 of timer TIM4 to start DMA transfer

TIM_DMACmd(TIM4, TIM_DMA_CC1, ENABLE);

/// --------------------------------------------------------------------------

///                      CONTRAST VOLTAGE GENERATOR enable

/// EG9018C contrast voltage generator is enabled by high level on pin PA2

/// --------------------------------------------------------------------------

#if (EG9018C_CNTR_GEN_SWD == 1)

// PA2 configuration as output

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_2;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_Out_PP;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_2MHz;

GPIO_Init(GPIOA, &GPIO_InitStructure);

// enable EG9018C contrast voltage generator

GPIO_SetBits(GPIOA, GPIO_Pin_2);

#endif

/// --------------------------------------------------------------------------

///                         Start servicing LCD

/// --------------------------------------------------------------------------

// Request the first DMA transfer

TIM_GenerateEvent(TIM4, TIM_EventSource_CC1);

// Enable TIM1 counter

TIM_Cmd(TIM1, ENABLE);

// Enable TIM5 counter

TIM_Cmd(TIM5, ENABLE);

// Enable TIM4  counter

TIM_Cmd(TIM4, ENABLE);

}

Listing 2: Funkcje realizujące zapalanie i gaszenie piksela

/**-----------------------------------------------------------------------------

Name         :  LCD_PixelOn

Description  :  Function draws pixel

Argument(s)  :  Xpos -> pixel x coordinate (0..639)

Ypos -> pixel y coordinate (0..479)

Return value :  none

------------------------------------------------------------------------------*/

void LCD_PixelOn(uint16_t Xpos, uint16_t Ypos)

{

uint8_t colNo, rowNo, bitNo;	// column number, row number and bit number in frame buffer

if (Xpos < EG9018C_XRES && Ypos < EG9018C_YRES){  // if pixel coordinates within range

// Calculate pixel position in LCD frame buffer

colNo = Xpos/4;

rowNo = (Ypos < EG9018C_YRES/2) ? Ypos : Ypos - EG9018C_YRES/2;

bitNo = (Ypos < EG9018C_YRES/2) ? 7 - Xpos%4 : 3 - Xpos%4;

// Draw pixel

// (acces to pixel bit is performed via bit-banding region of SRAM)

*(__IO uint32_t*)(SRAM_BB_BASE|(((uint32_t)&EG9018C_frame_buffer[rowNo][colNo]-SRAM_BASE)<<5)\

|((bitNo)<<2)) = LCD_PIXEL_ON;

}

}

/**-----------------------------------------------------------------------------

Name         :  LCD_PixelOff

Description  :  Function clears pixel

Argument(s)  :  Xpos -> pixel x coordinate (0..639)

Ypos -> pixel y coordinate (0..479)

Return value :  none

------------------------------------------------------------------------------*/

void LCD_PixelOff(uint16_t Xpos, uint16_t Ypos)

{

uint8_t colNo, rowNo, bitNo;	// column number, row number and bit number in frame buffer

if (Xpos < EG9018C_XRES && Ypos < EG9018C_YRES){  // if pixel coordinates within range

// Calculate pixel position in LCD frame buffer

colNo = Xpos/4;

rowNo = (Ypos < EG9018C_YRES/2) ? Ypos : Ypos - EG9018C_YRES/2;

bitNo = (Ypos < EG9018C_YRES/2) ? 7 - Xpos%4 : 3 - Xpos%4;

// Clear pixel

// (acces to pixel bit is performed via bit-banding region of SRAM)

*(__IO uint32_t*)(SRAM_BB_BASE|(((uint32_t)&EG9018C_frame_buffer[rowNo][colNo]-SRAM_BASE)<<5)\

|((bitNo)<<2)) = LCD_PIXEL_OFF;

}

}