Czytnik kart zbliżeniowych MIFARE – listingi - Adam Tatuś

/*Odczyt numeru karty */

#include <Mfrc522.h>

#include <SPI.h>

int chipSelectPin = 10;

int NRSTDP = 5;

Mfrc522 Mfrc522(chipSelectPin,NRSTDP);

unsigned char serNum[5];

void setup()

{

Serial.begin(9600);

SPI.begin();

digitalWrite(chipSelectPin, LOW);

}

void loop()

{

unsigned char i,tmp;

unsigned char status;

unsigned char str[MAX_LEN];

unsigned char RC_size;

unsigned char blockAddr;

String mynum = "";

status = Mfrc522.Request(PICC_REQIDL, str);

if (status == MI_OK)

{

status = Mfrc522.Anticoll(str);

memcpy(serNum, str, 5);

if (status == MI_OK)

{

Serial.print("Numer karty : ");

Serial.print(serNum[0],HEX); Serial.print(" , ");

Serial.print(serNum[1],HEX); Serial.print(" , ");

Serial.print(serNum[2],HEX); Serial.print(" , ");

Serial.print(serNum[3],HEX); Serial.print(" , ");

Serial.print(serNum[4],HEX); Serial.println(" HEX");

delay(1000);

}

}

Mfrc522.Halt();

}

Listing 1. Odczyt numeru karty

/*Prosty zamek otwierany zblizeniowo MSP430G2412*/

#include <Mfrc522.h>

#include <SPI.h>

int chipSelectPin = 10;

int NRSTDP = 5;

Mfrc522 Mfrc522(chipSelectPin,NRSTDP);

unsigned char serNum[5];

// numer karty otwierajacej rygiel

unsigned int card[5] = {0x3D,0xD7,0x93,0x19,0x60};

const int REL =  P2_3;

const int LED =  P2_4;

const int BUZ =  P2_5;

void setup()

{

SPI.begin();

digitalWrite(chipSelectPin, LOW);

pinMode(LED, OUTPUT);  digitalWrite(LED, LOW);

pinMode(BUZ, OUTPUT);  digitalWrite(BUZ, LOW);

pinMode(REL, OUTPUT);  digitalWrite(REL, LOW);

Mfrc522.Init();

}

void loop()

{

unsigned char i,tmp;

unsigned char status;

unsigned char str[MAX_LEN];

unsigned char RC_size;

unsigned char blockAddr;

String mynum = "";

status = Mfrc522.Request(PICC_REQIDL, str);

status = Mfrc522.Anticoll(str);

memcpy(serNum, str, 5);

if (status == MI_OK)

{ // sygnalizacja odczytu karty LED

digitalWrite(LED, HIGH);

delay(200);

digitalWrite(LED, LOW);

if(serNum[0] == card[0])

if (serNum[1] == card[1])

if (serNum[2] == card[2])

if (serNum[3] == card[3])

if (serNum[4] == card[4])

{ // numer karty zgodny otwarcie rygla + buzzer

digitalWrite(BUZ, HIGH);

digitalWrite(REL, HIGH);

delay(3000);

digitalWrite(BUZ, LOW);

digitalWrite(REL, LOW);

}

}

delay(500);

Mfrc522.Halt();

}

Listing 2. Prosty zamek RFID