Robert Wołgajew – Radio - listingi

//Software SPI port definitions #define SPI_PORT PORTC #define SPI_DDR DDRC #define MOSI_PIN PC0 #define SCK_PIN PC1 #define CS_PIN PC3 #define RESET_SCK SPI_PORT &= ~(1<<SCK_PIN) #define SET_SCK SPI_PORT ¦= (1<<SCK_PIN) #define RESET_MOSI SPI_PORT &= ~(1<<MOSI_PIN) #define SET_MOSI SPI_PORT ¦= (1<<MOSI_PIN) #define SCK_TICK SET_SCK; RESET_SCK #define RESET_CS SPI_PORT &= ~(1<<CS_PIN) #define SET_CS SPI_PORT ¦= (1<<CS_PIN) inline void SPIinit(void) { //MOSI, SCK & CS as outputs with “0” SPI_DDR ¦= (1<<MOSI_PIN)¦(1<<SCK_PIN)¦(1<<CS_PIN); } inline void SPIsendByte(uint8_t Byte) { //MSB first if(Byte&0x80) SET_MOSI; else RESET_MOSI; SCK_TICK; if(Byte&0x40) SET_MOSI; else RESET_MOSI; SCK_TICK; if(Byte&0x20) SET_MOSI; else RESET_MOSI; SCK_TICK; if(Byte&0x10) SET_MOSI; else RESET_MOSI; SCK_TICK; if(Byte&0x08) SET_MOSI; else RESET_MOSI; SCK_TICK; if(Byte&0x04) SET_MOSI; else RESET_MOSI; SCK_TICK; if(Byte&0x02) SET_MOSI; else RESET_MOSI; SCK_TICK; if(Byte&0x01) SET_MOSI; else RESET_MOSI; SCK_TICK; }

List.1 Ciała funkcji odpowiedzialnych za realizację programowej obsługi magistrali SPI.

void ST7565writeData(uint8_t Data) { SET_RS; SPIsendByte(Data); } void ST7565writeCommand(uint8_t Command) { RESET_RS; SPIsendByte(Command); }

List.2 Ciała funkcji odpowiedzialnych za realizację podstawowej komunikacji ze sterownikiem ST7565R.

#define COG_RS_PORT PORTC //COG display port definitions #define COG_RS_DDR DDRC #define COG_RST_PORT PORTD #define COG_RST_DDR DDRD #define COG_RS_NR PC2 //RS pin (1=Data, 0=Command) #define COG_RST_NR PD1 //RESET pin #define RESET_RS COG_RS_PORT &= ~(1<<COG_RS_NR) #define SET_RS COG_RS_PORT ¦= (1<<COG_RS_NR) #define RESET_RST COG_RST_PORT &= ~(1<<COG_RST_NR) #define SET_RST COG_RST_PORT ¦= (1<<COG_RST_NR) //Supported commands (CMD stands for “command”) and their possible settings (without “CMD”) #define CMD_DISPLAY_OFF 0xAE #define CMD_DISPLAY_ON 0xAF #define CMD_SET_DISP_START_LINE 0x40 #define CMD_SET_PAGE 0xB0 #define CMD_SET_COLUMN_MSB 0x10 #define CMD_SET_COLUMN_LSB 0x00 #define CMD_SET_ADC_NORMAL 0xA0 #define CMD_SET_ADC_REVERSE 0xA1 #define CMD_SET_DISP_NORMAL 0xA6 #define CMD_SET_DISP_REVERSE 0xA7 #define CMD_SET_ALL_PIXELS_NORMAL 0xA4 #define CMD_SET_ALL_PIXELS_ON 0xA5 #define CMD_SET_BIAS_1_9 0xA2 #define CMD_SET_BIAS_1_7 0xA3 #define CMD_READ_MODIFY_WRITE_START 0xE0 #define CMD_READ_MODIFY_WRITE_END 0xEE #define CMD_INTERNAL_RESET 0xE2 #define CMD_SET_COM_NORMAL 0xC0 #define CMD_SET_COM_REVERSE 0xC8 #define CMD_SET_POWER_CONTROL 0x28 #define BOOSTER_OFF (0<<2) #define BOOSTER_ON (1<<2) #define VOLTAGE_REGULATOR_OFF (0<<1) #define VOLTAGE_REGULATOR_ON (1<<1) #define VOLTAGE_FOLLOWER_OFF (0<<0) #define VOLTAGE_FOLLOWER_ON (1<<0) #define CMD_SET_V0_RESISTOR_RATIO 0x20 //0...7 #define CMD_SET_ELECTR_VOLUME_MODE 0x81 //0...3F #define CMD_SET_STATIC_INDICATOR_OFF 0xAC #define CMD_SET_STATIC_INDICATOR_ON 0xAD #define CMD_SET_BOOSTER_RATIO 0xF8 #define BOOSTER_234 0x00 #define BOOSTER_5 0x01 #define BOOSTER_6 0x03 #define CMD_NOP 0xE3 #define CMD_TEST 0xF0

List.3 Listing pliku nagłówkowego sterownika ST7565R.

void ST7565init(void) { SPIinit(); //MOSI, SCK & CS as outputs with '0' COG_RS_DDR ¦= (1<<COG_RS_NR); //RESET, RS as outputs with '0' COG_RST_DDR ¦= (1<<COG_RST_NR); RESET_RST; _delay_ms(5); SET_RST; _delay_ms(5); //Hardware RESET //Hardware configuration ST7565writeCommand(CMD_SET_ADC_REVERSE); //ADC Select = Reverse ST7565writeCommand(CMD_SET_COM_NORMAL); //COM scan direction = Normal ST7565writeCommand(CMD_SET_DISP_NORMAL); //LCD mode = Normal ST7565writeCommand(CMD_SET_ALL_PIXELS_NORMAL); //Pixels mode = Normal //Turn on voltage converter (VC=1, VR=0, VF=0) ST7565writeCommand(CMD_SET_POWER_CONTROL¦BOOSTER_ON); _delay_ms(50); //Turn on voltage regulator (VC=1, VR=1, VF=0) ST7565writeCommand(CMD_SET_POWER_CONTROL¦BOOSTER_ON¦VOLTAGE_REGULATOR_ON); _delay_ms(50); //Turn on voltage follower (VC=1, VR=1, VF=1) ST7565writeCommand(CMD_SET_POWER_CONTROL¦BOOSTER_ON¦VOLTAGE_REGULATOR_ON¦VOLTAGE_FOLLOWER_ON); _delay_ms(10); ST7565writeCommand(CMD_SET_BIAS_1_9); //LCD drive voltage bias = 1/9 ST7565writeCommand(CMD_SET_V0_RESISTOR_RATIO¦6); //Voltage Regulator Internal Resistor Ratio Set (0x00~0x07) ST7565writeCommand(CMD_SET_ELECTR_VOLUME_MODE); //Electronic volume mode set ST7565writeCommand(0x14); //0x01~0x3F ST7565writeCommand(CMD_SET_BOOSTER_RATIO); //Boster ratio select ST7565writeCommand(BOOSTER_234); //Booster ratio = 2x,3x,4x ST7565writeCommand(CMD_SET_DISP_START_LINE¦0); //Display start address = 0 ST7565writeCommand(CMD_SET_PAGE¦0); //Page = 0 ST7565writeCommand(CMD_SET_COLUMN_MSB¦0); //Column MSB = 0 ST7565writeCommand(CMD_SET_COLUMN_LSB¦0); //Column LSB = 0 ST7565writeCommand(CMD_DISPLAY_ON); //Display ON }

List.4 Ciało funkcji odpowiedzialnej za inicjalizację sterownika ST7565R.

void glcdGotoXY(uint8_t Column, uint8_t Page) { glcdColumn = Column; glcdPage = Page; //Variables glcdColumn/glcdPage are global variables of the module ST7565writeCommand(CMD_SET_PAGE¦(Page & 0x07)); //Page = 0~7 ST7565writeCommand(CMD_SET_COLUMN_MSB¦(Column>>4)); //Column A7~A4 ST7565writeCommand(CMD_SET_COLUMN_LSB¦(Column & 0x0F)); //Column A3~A0 } void glcdSetContrast(uint8_t Contrast) { ST7565writeCommand(CMD_SET_ELECTR_VOLUME_MODE); ST7565writeCommand(Contrast & 0x3F); } void glcdCls(void) { for(uint8_t Page = 0; Page < 8; Page++) { glcdGotoXY(0, Page); for(uint8_t Column = 0; Column<SCREEN_WIDTH; Column++) ST7565writeData(0x00); } }

List.5 Ciała funkcji narzędziowych sterownika ST7565R.

void glcdDrawBitmap(uint8_t X, uint8_t Y, const uint8_t *Bitmap) { register uint8_t Width, Height; Width = pgm_read_byte(Bitmap++); //The first byte is picture width (in pixels) Height = pgm_read_byte(Bitmap++)>>3; //The second byte is picture height (in pixels). Must be 8, 16, 24 etc. for(uint8_t Page = 0; Page < Height; Page++) { glcdGotoXY(X, Y+Page); for(uint8_t Column = 0; Column < Width; Column++) ST7565writeData(pgm_read_byte(Bitmap++)); } }

List.6 Ciało funkcji odpowiedzialnej za wyświetlenie obrazka na ekranie wyświetlacza LCD.

typedef struct { uint8_t Width; //Font width (pixels) uint8_t Height; //Font height (bytes!) uint8_t Interspace; //Interspace width (pixels) uint8_t BytesPerChar; //Bytes per character definition uint8_t FirstCharCode; //The ASCII code of the first character in the font data array const uint8_t *Bitmap; //Pointer to the array describing subsequent characters } fontDescription; static fontDescription CurrentFont

List.7 Struktura danych (i deklaracja stosownej zmiennej) przechowująca parametry aktualnie używanej czcionki.

void glcdSetFont(const fontDescription *Font) { CurrentFont.Width = pgm_read_byte(&Font->Width); CurrentFont.Height = pgm_read_byte(&Font->Height); CurrentFont.Interspace = pgm_read_byte(&Font->Interspace); CurrentFont.BytesPerChar = pgm_read_byte(&Font->BytesPerChar); CurrentFont.FirstCharCode = pgm_read_byte(&Font->FirstCharCode); CurrentFont.Bitmap = (uint8_t*)pgm_read_word(&Font->Bitmap); }

List.8 Ciało funkcji narzędziowej odpowiedzialnej za ustawienie parametrów aktualnej czcionki ekranowej.

void glcdDrawChar(char Character, uint8_t Inversion) { register uint8_t readByte, positionX = glcdColumn, positionY = glcdPage; const uint8_t *dataPointer; //Now we calculate pointer to the character’s data dataPointer = &CurrentFont.Bitmap[(CurrentFont.BytesPerChar*(Character-CurrentFont.FirstCharCode))]; for(uint8_t Page = 0; Page < CurrentFont.Height; Page++) { glcdGotoXY(positionX, positionY++); for(uint8_t Column = 0; Column < CurrentFont.Width; Column++) { if(Character == ' ') readByte = 0x00; else readByte = pgm_read_byte(dataPointer++); ST7565writeData(readByte); } } }

List.9 Ciało funkcji odpowiedzialnej za wyświetlenie pojedynczego znaku aktualnie używaną czcionką.

void glcdDrawString(uint8_t Column, uint8_t Page, char *String) { glcdGotoXY(Column, Page); while(*String) { glcdDrawChar(*String++, Inversion); //Now we have to calculate the new column address where the next character will be displayed glcdColumn += CurrentFont.Width + CurrentFont.Interspace; } }

List.10 Ciało funkcji odpowiedzialnej za wyświetlenie ciągu znaków z pamięci RAM mikrokontrolera.