Listingi - komputer samochodowy Mee MK II - Robert Wołgajew

void writeCommand(uint8_t Command) { RESET_DCX; //Command RESET_WRX; TFT_DATA_PORT = Command; SET_WRX; //TFT reads data at the rising edge SET_DCX; //DCX is 1 by default to increase DRAM data transfer } void writeData(uint8_t Data) { RESET_WRX; TFT_DATA_PORT = Data; SET_WRX; //TFT reads data at the rising edge }

List.1 Podstawowe funkcje narzędziowe odpowiedzialne za wysyłanie rozkazów sterujących lub danych do wyświetlacza TFT.

void TFTinit(void) { TFT_DATA_DDR = 0xFF; //Data port as output with 0x00 TFT_CTRL_PORT¦= (1<<WRX_PIN)¦(1<<DCX_PIN)¦(1<<RESX_PIN)¦(1<<RDX_PIN); //ChipSelect (CSX) is 0 by default TFT_CTRL_DDR ¦= (1<<WRX_PIN)¦(1<<CSX_PIN)¦(1<<DCX_PIN)¦(1<<RESX_PIN)¦(1<<RDX_PIN); //All control ports are outputs _delay_ms(5); //Hardware reset RESET_RESX; _delay_ms(5); SET_RESX; _delay_ms(50); //Software reset and configuration writeCommand(CMD_SOFTWARE_RESET); _delay_ms(5); writeCommand(CMD_DISPLAY_OFF); writeCommand(CMD_PWR_CTRL_B); writeData(0x00); //Default values writeData(0x83); writeData(0x30); writeCommand(CMD_PWR_ON_SEQ_CTRL); writeData(0x64); //Default values writeData(0x03); writeData(0x12); writeData(0x81); writeCommand(CMD_DRIVER_TIM_CTRL_A); writeData(0x85); //Default values writeData(0x01); writeData(0x79); writeCommand(CMD_PWR_CTRL_A); writeData(0x39); //Default value writeData(0x2c); //Default value writeData(0x00); //Default value writeData(VCORE_1_60); writeData(DDVDH_5_6); writeCommand(CMD_PUMP_RATIO_CTRL); writeData(0x20); //Default value writeCommand(CMD_DRIVER_TIM_CTRL_B); writeData(0x00); //Default value writeData(0x00); //Default value //Power Control writeCommand(CMD_PWR_CTRL_1); writeData(0x26); //Default value writeCommand(CMD_PWR_CTRL_2); writeData(0x11); //Default value //VCOM setting writeCommand(CMD_VCOM_CTRL_1); writeData(0x35); //Default value writeData(0x3e); //Default value writeCommand(CMD_VCOM_CTRL_2); writeData(0xbe); //Default value //Memory Access Control writeCommand(CMD_MEM_ACCESS_CTRL); writeData(ROW_COLUMN_EXCHANGE_ON¦RGB_ORDER_BGR); writeCommand(CMD_PIXEL_FORMAT); writeData(RGB_INTERFACE_16BITS¦MCU_INTERFACE_16BITS); //Frame Rate writeCommand(CMD_FRAME_RATE); writeData(DIVISION_RATIO_FOSC); writeData(FRAME_RATE_119KHZ); //Gamma writeCommand(CMD_GAMMA3_FUNCTION_DISABLE); writeData(0x08); //Default value writeCommand(CMD_SET_GAMMA_CURVE_NR); writeData(0x01); //One curve writeCommand(CMD_POSITIVE_GAMMA_CORR); writeData(0x1f); writeData(0x1a); writeData(0x18); writeData(0x0a); writeData(0x0f); writeData(0x06); writeData(0x45); writeData(0x87); writeData(0x32); writeData(0x0a); writeData(0x07); writeData(0x02); writeData(0x07); writeData(0x05); writeData(0x00); writeCommand(CMD_NEGATIVE_GAMMA_CORR); writeData(0x00); writeData(0x25); writeData(0x27); writeData(0x05); writeData(0x10); writeData(0x09); writeData(0x3a); writeData(0x78); writeData(0x4d); writeData(0x05); writeData(0x18); writeData(0x0d); writeData(0x38); writeData(0x3a); writeData(0x1f); //DISPLAY RAM writeCommand(CMD_COLUMN_ADDR_SET); writeData(0x00); writeData(0x00); writeData((TFT_WIDTH-1)>>8); writeData((uint8_t) TFT_WIDTH-1); writeCommand(CMD_PAGE_ADDR_SET); writeData(0x00); writeData(0x00); writeData((TFT_HEIGHT-1)>>8); writeData(TFT_HEIGHT-1); writeCommand(CMD_ENTRY_MODE); writeData(GON_DTE_NORMAL_DISPLAY¦LOW_VOLTAGE_DETECT_DISBLE); //Display writeCommand(CMD_DISPLAY_FUNCTION_CTRL); writeData(INTERVAL_SCAN¦0b10); writeData(LCD_NORMALLY_WHITE¦GATE_OUT_SCAN_DIR_NORMAL¦SOURCE_OUT_SCAN_DIR_NORMAL¦GATE_OUT_SEQUENCE_NORMAL¦SCAN_CYCLE_INTERVAL_5FRAME); writeData(NUMBER_OF_LINES_320); writeData(0x00); //Default value writeCommand(CMD_SLEEP_OUT); _delay_ms(5); writeCommand(CMD_DISPLAY_ON); _delay_ms(5); }

List.2 Funcja inicjalizacyjna sterownika ekranu ILI9341 wyświetlacza Rivierdi RVT28AETNWN00.

//PHYSICAL PARAMETERS - LANDSCAPE MODE #define TFT_WIDTH 320 #define TFT_HEIGHT 240 //PORTS CONFIGURATION. We assume that RDX (read signal) is permanently pulled high #define TFT_DATA_PORT PORTA #define TFT_DATA_DDR DDRA #define TFT_CTRL_PORT PORTB #define TFT_CTRL_DDR DDRB #define WRX_PIN PB1 //Write signal #define RDX_PIN PB7 //Read signal #define CSX_PIN PB3 //Chip select signal #define DCX_PIN PB2 //Command/Data signal: 1->DRAM data, 0->Command #define RESX_PIN PB4 //Reset signal //MACROS #define SET_WRX TFT_CTRL_PORT ¦= (1<<WRX_PIN) #define RESET_WRX TFT_CTRL_PORT &= ~(1<<WRX_PIN) #define SET_CSX TFT_CTRL_PORT ¦= (1<<CSX_PIN) #define RESET_CSX TFT_CTRL_PORT &= ~(1<<CSX_PIN) #define SET_DCX TFT_CTRL_PORT ¦= (1<<DCX_PIN) #define RESET_DCX TFT_CTRL_PORT &= ~(1<<DCX_PIN) #define SET_RESX TFT_CTRL_PORT ¦= (1<<RESX_PIN) #define RESET_RESX TFT_CTRL_PORT &= ~(1<<RESX_PIN) //ILI9341 COMMANDS AND PARAMETERS #define CMD_SOFTWARE_RESET 0x01 //5ms delay is needed after this command #define CMD_SLEEP_OUT 0x11 //5ms delay is needed after this command #define CMD_SET_GAMMA_CURVE_NR 0x26 //Selects gamma curve number #define CMD_DISPLAY_OFF 0x28 #define CMD_DISPLAY_ON 0x29 #define CMD_COLUMN_ADDR_SET 0x2A //Sets current column address #define CMD_PAGE_ADDR_SET 0x2B //Sets current page address #define CMD_MEMORY_WRITE 0x2C //Starts frame memory writting #define CMD_MEM_ACCESS_CTRL 0x36 //Defines read/write scanning direction of frame memory #define ROW_ADDR_ORDER_NORMAL (0<<7) #define ROW_ADDR_ORDER_REVERSE (1<<7) #define COLUMN_ADDR_ORDER_NORMAL (0<<6) #define COLUMN_ADDR_ORDER_REVERSE (1<<6) #define ROW_COLUMN_EXCHANGE_OFF (0<<5) #define ROW_COLUMN_EXCHANGE_ON (1<<5) #define VERTICAL_REFRESH_ORDER_NORMAL (0<<4) #define VERTICAL_REFRESH_ORDER_REVERSE (1<<4) #define RGB_ORDER_RGB (0<<3) #define RGB_ORDER_BGR (1<<3) #define HORIZONTAL_REFRESH_ORDER_NORMAL (0<<2) #define HORIZONTAL_REFRESH_ORDER_REVERSE (1<<2) #define CMD_PIXEL_FORMAT 0x3A //Sets pixel data format #define RGB_INTERFACE_16BITS (0b101<<4) #define RGB_INTERFACE_18BITS (0b110<<4) #define MCU_INTERFACE_16BITS (0b101) #define MCU_INTERFACE_18BITS (0b110) #define CMD_FRAME_RATE 0xB1 //Sets the division ratio for internal clocks #define DIVISION_RATIO_FOSC 0x00 //1st parameter #define DIVISION_RATIO_FOSC_2 0x01 #define DIVISION_RATIO_FOSC_4 0x02 #define DIVISION_RATIO_FOSC_8 0x03 #define FRAME_RATE_119KHZ 0b10000 //2nd parameter #define FRAME_RATE_112KHZ 0b10001 #define FRAME_RATE_106KHZ 0b10010 #define FRAME_RATE_100KHZ 0b10011 #define FRAME_RATE_95KHZ 0b10100 #define FRAME_RATE_90KHZ 0b10101 #define FRAME_RATE_85KHZ 0b10110 #define FRAME_RATE_83KHZ 0b10111 #define CMD_DISPLAY_FUNCTION_CTRL 0xB6 #define NORMAL_SCAN (0b00<<2) //1st parameter #define INTERVAL_SCAN (0b10<<2) #define LCD_NORMALLY_BLACK (0<<7) //2nd parameter #define LCD_NORMALLY_WHITE (1<<7) #define GATE_OUT_SCAN_DIR_NORMAL (0<<6) #define GATE_OUT_SCAN_DIR_REVERSE (1<<6) #define SOURCE_OUT_SCAN_DIR_NORMAL (0<<5) #define SOURCE_OUT_SCAN_DIR_REVERSE (1<<5) #define GATE_OUT_SEQUENCE_NORMAL (0<<4) #define GATE_OUT_SEQUENCE_REVERSE (1<<4) #define SCAN_CYCLE_INTERVAL_1FRAME (0b0000) #define SCAN_CYCLE_INTERVAL_3FRAME (0b0001) #define SCAN_CYCLE_INTERVAL_5FRAME (0b0010) #define SCAN_CYCLE_INTERVAL_7FRAME (0b0011) #define SCAN_CYCLE_INTERVAL_9FRAME (0b0100) #define SCAN_CYCLE_INTERVAL_11FRAME (0b0101) #define SCAN_CYCLE_INTERVAL_13FRAME (0b0110) #define SCAN_CYCLE_INTERVAL_15FRAME (0b0111) #define NUMBER_OF_LINES_320 0x27 //3rd parameter #define NUMBER_OF_LINES_240 0x1D #define CMD_ENTRY_MODE 0xB7 #define LOW_VOLTAGE_DETECT_ENABLE (0x00) #define LOW_VOLTAGE_DETECT_DISBLE (0x01) #define GON_DTE_VGH (0x00<<1) #define GON_DTE_VGH2 (0x01<<1) #define GON_DTE_VGL (0x02<<1) #define GON_DTE_NORMAL_DISPLAY (0x03<<1) #define CMD_PWR_CTRL_1 0xC0 //Sets the GVDD level, which is a reference level for the VCOM #define CMD_PWR_CTRL_2 0xC1 //Sets the factor used in the step-up circuits. #define CMD_VCOM_CTRL_1 0xC5 //Sets the VCOM voltage. #define CMD_VCOM_CTRL_2 0xC7 //Set the VCOM offset voltage #define CMD_PWR_CTRL_A 0xCB #define VCORE_1_55 0x30 #define VCORE_1_40 0x31 #define VCORE_1_50 0x32 #define VCORE_1_65 0x33 #define VCORE_1_60 0x34 #define VCORE_1_70 0x35 #define DDVDH_5_8 0x00 #define DDVDH_5_7 0x01 #define DDVDH_5_6 0x02 #define DDVDH_5_5 0x03 #define DDVDH_5_4 0x04 #define DDVDH_5_3 0x05 #define DDVDH_5_2 0x06 #define CMD_PWR_CTRL_B 0xCF #define CMD_POSITIVE_GAMMA_CORR 0xE0 //Set the gray scale voltage to adjust the gamma of the TFT #define CMD_NEGATIVE_GAMMA_CORR 0xE1 //Set the gray scale voltage to adjust the gamma of the TFT #define CMD_DRIVER_TIM_CTRL_A 0xE8 //EQ timing for Internal clock #define CMD_DRIVER_TIM_CTRL_B 0xEA #define CMD_PWR_ON_SEQ_CTRL 0xED #define CMD_GAMMA3_FUNCTION_DISABLE 0xF2 #define CMD_PUMP_RATIO_CTRL 0xF7 //Auxiliary macros - used in selected functions #define SOLID_TEXT 0 #define TRANSPARENT_TEXT 1

List.3 Plik nagłówkowy sterownika ekranu ILI9341 wyświetlacza Rivierdi RVT28AETNWN00.

void TFTsetActiveWindow(uint16_t X1, uint8_t Y1, uint16_t X2, uint8_t Y2) { writeCommand(CMD_COLUMN_ADDR_SET); writeData(X1 >> 8); writeData(X1 & 0xFF); writeData(X2 >> 8); writeData(X2 & 0xFF); writeCommand(CMD_PAGE_ADDR_SET); writeData(Y1 >> 8); writeData(Y1); writeData(Y2 >> 8); writeData(Y2); }

List.4 Funkcja narzędziowa odpowiedzialna za ustawienie aktywnego obszaru ekranu, w ramach którego przeprowadzany jest zapis do pamięci ekranu sterownika ILI9341

void TFTdrawFilledBox(uint16_t X1, uint8_t Y1, uint16_t X2, uint8_t Y2) { uint32_t pixelsToSend = (X2-X1+1)*(Y2-Y1+1); //We calculate how many pixels we need to send (2 bytes/pixel) //We define display active area to simplify writing TFTsetActiveWindow(X1, Y1, X2, Y2); //We start memory writing writeCommand(CMD_MEMORY_WRITE); while(pixelsToSend --) {writeData(Colour>>8); writeData(Colour&0xFF);} //(2 bytes/pixel) } void TFTdrawRectangle(uint16_t X1, uint8_t Y1, uint16_t X2, uint8_t Y2) { TFTdrawFilledBox(X1 , Y1 , X2 , Y1); TFTdrawFilledBox(X1 , Y2 , X2 , Y2); TFTdrawFilledBox(X1 , Y1 , X1 , Y2); TFTdrawFilledBox(X2 , Y1 , X2 , Y2); }

List.5 Funkcje umożliwiające wyświetlanie wypełnionego i „pustego” prostokąta na ekranie wyświetlacza TFT.

void TFTdrawPicture(uint16_t X1, uint8_t Y1, const uint16_t *Picture) { register uint16_t pixelData, pixelsToSend; register uint8_t Width, Height; //We read the first word that holds the picture width and height (MSB and LSB) pixelData = pgm_read_word(Picture++); //We calculate the picture width and height Width = pixelData >> 8; Height = pixelData & 0xFF; //We calculate how many pixels we need to send pixelsToSend = Width * Height; //We define display active area to simplify writing TFTsetActiveWindow(X1, Y1, X1+Width-1, Y1+Height-1); //We start memory writing writeCommand(CMD_MEMORY_WRITE); while(pixelsToSend--) { pixelData = pgm_read_word(Picture++); writeData(pixelData >> 8); writeData(pixelData & 0xFF); } }

List.6 Funkcja odpowiedzialna za wyświetlanie obrazków na ekranie wyświetlacza TFT.

void TFTdrawCompressedPicture(uint16_t X1, uint8_t Y1, const uint16_t *Picture) { register uint16_t pixelsToSend, pixelA, pixelB; register uint8_t Width, Height; //We read the first word that holds the picture width and height (MSB and LSB) pixelA = pgm_read_word(Picture++); //We calculate the picture width and height Width = pixelA >> 8; Height = pixelA & 0xFF; //We calculate how many pixels we need to send pixelsToSend = Width * Height; //We define display active area to simplify writing TFTsetActiveWindow(X1, Y1, X1+Width-1, Y1+Height-1); //We start memory writing writeCommand(CMD_MEMORY_WRITE); while(pixelsToSend) { //We read pixel n and n+1 pixelA = pgm_read_word(Picture++); pixelB = pgm_read_word(Picture); //If the pixel n is different than the pixel n+1 or if it is the last pixel we send it to the TFT if(pixelA != pixelB ¦¦ pixelsToSend == 1) { writeData(pixelA >> 8); writeData(pixelA & 0xFF); pixelsToSend--; } else { //Otherwise we read how many the same pixels we need to send (third word) pixelB = pgm_read_word(++Picture); pixelsToSend -= pixelB; Picture++; //We sent them to the TFT while(pixelB--) {writeData(pixelA >> 8); writeData(pixelA & 0xFF);} } } }

List.7 Funkcja odpowiedzialna za wyświetlanie skompresowanych obrazków na ekranie wyświetlacza TFT.

//Deklaracja struktury przechowującej parametry bieżącej czcionki ekranowej typedef struct { uint8_t Width; //Current font widht (px) uint8_t Height; //Current font height (px) uint8_t Interspace; //Font interspace (px) uint8_t BytesPerChar; //Bytes per char definition uint8_t FirstCharCode; //First char ASCII code const uint8_t *Bitmap; //Pointer to the font table } fontDescription;

List.8 Definicja nowego typu danych odpowiedzialnego za przechowywanie parametrów bieżącej czcionki ekranowej.

void OLEDsetFont(constfontDescription *Font) { CurrentFont.Width = pgm_read_byte(&Font->Width); //Rzeczywista szerokość czcionki CurrentFont.Height = pgm_read_byte(&Font->Height); //Rzeczywista wysokość czcionki CurrentFont.Interspace = pgm_read_byte(&Font->Interspace); //Odstęp pomiędzy znakami CurrentFont.BytesPerChar = pgm_read_byte(&Font->BytesPerChar); //Ilość bajtów na definicję pojedyńczego znaku CurrentFont.FirstCharCode = pgm_read_byte(&Font->FirstCharCode); //Kod ASCII definicji pierwszego znaku CurrentFont.Bitmap = (uint8_t*)pgm_read_word(&Font->Bitmap); //Wskaźnik do tablicy wzorców tej czcionki }

List.9 Funkcja odpowiedzialna za ustawienie bieżącej czcionki ekranowej.

void TFTdrawChar(uint16_t X1, uint8_t Y1, char Character, const uint8_t Transparency) { register uint8_t widthIndex, heightIndex, readByte, pixelsNr, i; const uint8_t *dataPointer; if(Transparency != TRANSPARENT_TEXT) { //We define display active area to simplify writing TFTsetActiveWindow(X1, Y1, X1+CurrentFont.Width-1, Y1+CurrentFont.Height-1); //We start memory writing writeCommand(CMD_MEMORY_WRITE); } //Now we calculate start address of the current character definition dataPointer = &CurrentFont.Bitmap[(CurrentFont.BytesPerChar*(Character-CurrentFont.FirstCharCode))]; for(heightIndex = 0; heightIndex < CurrentFont.Height; heightIndex++) { for(widthIndex = 0; widthIndex < CurrentFont.Width; widthIndex += 8) { //We read character definition byte by byte readByte = pgm_read_byte(dataPointer++); //For fonts which width is not a multiple of 8 we need to calculate useful number of pixels to be sent pixelsNr = widthIndex+8 <= CurrentFont.Width ? 8 : CurrentFont.Width - widthIndex; for(i=0; i<pixelsNr; ++i) { if(Transparency == TRANSPARENT_TEXT) //We check if the text background is transparent { //We check the pixel presence if(readByte & 0x80) { //We define display active area for one active pixel to simplify writing TFTsetActiveWindow(X1+widthIndex+i, Y1+heightIndex, X1+widthIndex+i, Y1+heightIndex); writeCommand(CMD_MEMORY_WRITE); writeData(Colour >> 8); writeData(Colour & 0xFF); } } else { //Pixel color depends on the pixel presence if(readByte & 0x80) {writeData(Colour >> 8); writeData(Colour & 0xFF);} else {writeData(Background >> 8); writeData(Background & 0xFF);} } readByte<<=1; } } } }

List.10 Funkcja odpowiedzialna za rysowanie znaków, przy użyciu bieżącej czcionki ekranowej.

void TFTdrawString(uint16_t X1, uint8_t Y1, char *String, const uint8_t Transparency) { while(*String) { TFTdrawChar(X1, Y1, *String++, Transparency); X1 += CurrentFont.Width + CurrentFont.Interspace; } } void TFTdrawString_P(uint16_t X1, uint8_t Y1, const char *String, const uint8_t Transparency) { register char Character; while((Character = pgm_read_byte(String++))) { TFTdrawChar(X1, Y1, Character, Transparency); X1 += CurrentFont.Width + CurrentFont.Interspace; } }

List.11 Funkcje umożliwiające wyświetlenie ciągu znaków z pamięci RAM, jak i pamięci programu (Flash).