Listing 1. Ramki danych w standardzie NMEA-0183

$GPRMC,125523.00,A,1234.56789,N,01234.56789,E,1.395,,170417,,,A*75

$GPVTG,,T,,M,1.395,N,2.583,K,A*21

$GPGGA,125523.00,1234.56789,N,01234.56789,E,1,06,1.75,292.2,M,42.0,M,,*5B

$GPGSA,A,3,24,19,32,17,06,02,,,,,,,4.63,1.75,4.29*02

$GPGSV,4,1,14,02,10,135,25,04,,,10,06,21,096,29,10,02,277,*4F

$GPGSV,4,2,14,11,,,08,13,,,13,14,,,20,15,,,08*79

$GPGSV,4,3,14,16,,,09,17,25,045,39,19,42,058,33,24,78,158,10*48

$GPGSV,4,4,14,25,27,259,14,32,26,311,10*72

$GPGLL,1234.56789,N,01234.56789,E,125523.00,A,A*66

Listing 2. Kod źródłowy pliku main.c

/* USER CODE BEGIN Includes */

#include "gps.h"

#include "string.h"

/* USER CODE END Includes */

/* USER CODE BEGIN 0 */

volatile struct gps_state gps_handle;

volatile uint8_t recv_char;

void HAL_UART_RxCpltCallback(UART_HandleTypeDef * uart) {

if (uart == &huart1) {

gps_recv_char(&gps_handle, recv_char);

HAL_UART_Receive_IT(&huart1, &recv_char, 1);

}

}

/* USER CODE END 0 */

int main(void)

{

...

/* USER CODE BEGIN 2 */

gps_handle = gps_init(&huart1);

HAL_UART_Receive_IT(&huart1, &recv_char, 1);

char output_buffer[100];

for (uint8_t i = 0; i < 100; i++) output_buffer[i] = '\0';

/* USER CODE END 2 */

/* Infinite loop */

/* USER CODE BEGIN WHILE */

while (1) {

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

sprintf(output_buffer, "\r\n");

HAL_UART_Transmit(&huart2, output_buffer, strlen(output_buffer), 100);

sprintf(output_buffer, "Data: %02d-%02d-20%02d\r\n", gps_handle.date_day, gps_handle.date_mounth, gps_handle.date_year);

HAL_UART_Transmit(&huart2, output_buffer, strlen(output_buffer), 100);

sprintf(output_buffer, "Czas: %02d:%02d:%02d\r\n", gps_handle.time_hour, gps_handle.time_min, gps_handle.time_sec);

HAL_UART_Transmit(&huart2, output_buffer, strlen(output_buffer), 100);

sprintf(output_buffer, "Szerokosc geograficzna: %f %c\r\n", gps_handle.latitude, gps_handle.latitude_direction);

HAL_UART_Transmit(&huart2, output_buffer, strlen(output_buffer), 100);

sprintf(output_buffer, "Dlugosc geograficzna: %f %c\r\n", gps_handle.longitude, gps_handle.longitude_direction);

HAL_UART_Transmit(&huart2, output_buffer, strlen(output_buffer), 100);

sprintf(output_buffer, "Wysokosc: %f m n.p.m.\r\n", gps_handle.altitude);

HAL_UART_Transmit(&huart2, output_buffer, strlen(output_buffer), 100);

sprintf(output_buffer, "Predkosc w wezlach: %f\r\n", gps_handle.speed_knots);

HAL_UART_Transmit(&huart2, output_buffer, strlen(output_buffer), 100);

sprintf(output_buffer, "Predkosc w km/h: %f\r\n", gps_handle.speed_kilometers);

HAL_UART_Transmit(&huart2, output_buffer, strlen(output_buffer), 100);

sprintf(output_buffer, "Liczba widocznych satelit: %d\r\n", gps_handle.satelites_number);

HAL_UART_Transmit(&huart2, output_buffer, strlen(output_buffer), 100);

sprintf(output_buffer, "Jakosc okreslonej pozycji: %d\r\n", gps_handle.quality);

HAL_UART_Transmit(&huart2, output_buffer, strlen(output_buffer), 100);

sprintf(output_buffer, "Precyzja wyznaczenia pozycji (DOP): %f\r\n", gps_handle.dop);

HAL_UART_Transmit(&huart2, output_buffer, strlen(output_buffer), 100);

sprintf(output_buffer, "Horyzontalna precyzja wyznaczenia pozycji (HDOP): %f\r\n", gps_handle.hdop);

HAL_UART_Transmit(&huart2, output_buffer, strlen(output_buffer), 100);

sprintf(output_buffer, "Wertykalna precyzja wyznaczenia pozycji (VDOP): %f\r\n", gps_handle.vdop);

HAL_UART_Transmit(&huart2, output_buffer, strlen(output_buffer), 100);

HAL_Delay(1000);

}

/* USER CODE END 3 */

}

Listing 3. Kod źródłowy pliku gps.h

#ifndef gps_header

#define gps_header

#include "stm32f4xx_hal.h"

#include "usart.h"

#include "stdlib.h"

#include "string.h"

struct gps_state

{

UART_HandleTypeDef * uart;

uint8_t line_buffer[100];

uint8_t writer_position;

uint8_t reader_position;

uint8_t field_buffer[30];

uint8_t field_position;

uint8_t date_day;

uint8_t date_mounth;

uint8_t date_year;

uint8_t time_hour;

uint8_t time_min;

uint8_t time_sec;

double latitude;

char latitude_direction;

double longitude;

char longitude_direction;

double altitude;

double speed_knots;

double speed_kilometers;

uint8_t satelites_number;

uint8_t quality;

double dop;

double hdop;

double vdop;

};

struct gps_state gps_init(UART_HandleTypeDef * _uart);

void gps_recv_char(struct gps_state * state, uint8_t recv_char);

void gps_read_field(struct gps_state * state);

void gps_process_line(struct gps_state * state);

void gps_process_gprmc(struct gps_state * state);

void gps_process_gpvtg(struct gps_state * state);

void gps_process_gpgga(struct gps_state * state);

void gps_process_gpgsa(struct gps_state * state);

#endif

Listing 4. Kod źródłowy pliku gps.c

#include "gps.h"

struct gps_state gps_init(UART_HandleTypeDef * uart)

{

struct gps_state state;

state.uart = uart;

for(uint8_t i=0; i<100; i++) state.line_buffer[i] = '\0';

state.writer_position = 0;

state.reader_position = 0;

for(uint8_t i=0; i<30; i++) state.field_buffer[i] = '\0';

state.field_position = 0;

state.date_day = 0;

state.date_mounth = 0;

state.date_year = 0;

state.time_hour = 0;

state.time_min = 0;

state.time_sec = 0;

state.latitude = 0.0;

state.latitude_direction = '?';

state.longitude = 0.0;

state.longitude_direction = '?';

state.altitude = 0.0;

state.speed_knots = 0.0;

state.speed_kilometers = 0.0;

state.satelites_number = 0;

state.quality = '?';

state.dop = 0.0;

state.hdop = 0.0;

state.vdop = 0.0;

return state;

}

void gps_recv_char(struct gps_state * state, uint8_t recv_char)

{

if (state->writer_position == 0 && recv_char == '$')

{

state->writer_position++;

} else if (state->writer_position >= 1 && state->writer_position < 99)

{

if (recv_char == '\r' || recv_char == '\n')

{

state->line_buffer[state->writer_position - 1] = '\0';

state->writer_position = 0;

gps_process_line(state);

} else

{

state->line_buffer[state->writer_position - 1] = recv_char;

state->writer_position++;

}

} else

{

state->writer_position = 0;

}

}

void gps_read_field(struct gps_state * state)

{

state->field_position = 0;

while(state->line_buffer[state->reader_position] != ',' && state->line_buffer[state->reader_position] != '\0'

&& state->field_position < 29)

{

state->field_buffer[state->field_position] = state->line_buffer[state->reader_position];

state->reader_position++;

state->field_position++;

}

state->field_buffer[state->field_position] = '\0';

state->reader_position++;

}

void gps_process_line(struct gps_state * state)

{

state->reader_position = 0;

gps_read_field(state);

if(strcmp(state->field_buffer, "GPRMC") == 0) gps_process_gprmc(state);

else if(strcmp(state->field_buffer, "GPVTG") == 0) gps_process_gpvtg(state);

else if(s trcmp(state->field_buffer, "GPGGA") == 0) gps_process_gpgga(state);

else if(strcmp(state->field_buffer, "GPGSA") == 0) gps_process_gpgsa(state);

}

void gps_process_gprmc(struct gps_state * state)

{

//$GPRMC,212846.00,A,5025.81511,N,01639.92090,E,0.196,,140417,,,A*73

gps_read_field(state);

if(strlen(state->field_buffer) > 0)

{

uint32_t tmp;

sscanf(state->field_buffer, "%d", &tmp);

state->time_sec = tmp % 100;

state->time_min = (tmp / 100) % 100;

state->time_hour = (tmp / 10000) % 100;

}

gps_read_field(state);

gps_read_field(state);

if(strlen(state->field_buffer) > 0) sscanf(state->field_buffer, "%lf", &(state->latitude));

gps_read_field(state);

if(strlen(state->field_buffer) > 0) sscanf(state->field_buffer, "%c", &(state->latitude_direction));

gps_read_field(state);

if(strlen(state->field_buffer) > 0) sscanf(state->field_buffer, "%lf", &(state->longitude));

gps_read_field(state);

if(strlen(state->field_buffer) > 0) sscanf(state->field_buffer, "%c", &(state->longitude_direction));

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

if(strlen(state->field_buffer) > 0) {

uint32_t tmp;

sscanf(state->field_buffer, "%d", &tmp);

state->date_year = tmp % 100;

state->date_mounth = (tmp / 100) % 100;

state->date_day = (tmp / 10000) % 100;

}

}

void gps_process_gpvtg(struct gps_state * state)

{

//$GPVTG,,T,,M,0.196,N,0.363,K,A*2B

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

if(strlen(state->field_buffer) > 0) sscanf(state->field_buffer, "%lf", &(state->speed_knots));

gps_read_field(state);

gps_read_field(state);

if(strlen(state->field_buffer) > 0) sscanf(state->field_buffer, "%lf", &(state->speed_kilometers));

}

void gps_process_gpgga(struct gps_state * state)

{

//$GPGGA,212846.00,5025.81511,N,01639.92090,E,1,04,4.72,281.1,M,42.0,M,,*5F

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

if(strlen(state->field_buffer) > 0) sscanf(state->field_buffer, "%d", &(state->quality));

gps_read_field(state);

if(strlen(state->field_buffer) > 0) sscanf(state->field_buffer, "%d", &(state->satelites_number));

gps_read_field(state);

gps_read_field(state);

if(strlen(state->field_buffer) > 0) sscanf(state->field_buffer, "%lf", &(state->altitude));

}

void gps_process_gpgsa(struct gps_state * state)

{

//$GPGSA,A,3,10,18,21,15,,,,,,,,,6.79,4.72,4.89*01

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

gps_read_field(state);

if(strlen(state->field_buffer) > 0) sscanf(state->field_buffer, "%lf", &(state->dop));

gps_read_field(state);

if(strlen(state->field_buffer) > 0) sscanf(state->field_buffer, "%lf", &(state->hdop));

gps_read_field(state);

if(strlen(state->field_buffer) > 0) sscanf(state->field_buffer, "%lf", &(state->vdop));

}