Listing 1. Procedura inicjowania akwiyzcji danych z panelu przy użyciu DMA

#define CHX  (0x90 >> 3)

#define CHY (0xd0 >> 3)

#define TP_TxDMACh	DMA1_Channel5

#define TP_RxDMACh	DMA1_Channel4

#define CLR_TPtx_FLAGS	(DMA1->IFCR = DMA_IFCR_CGIF4)

static uint16_t tp_rxbuf[3];

static void tp_hw_start_acquire(void)

{

static const uint16_t cmd[3] = {CHY, CHX, 0};

if (!TP_RxDMACh->CCR)

{

// rx - setup once, circular mode

DMA1_CSELR->CSELR = BF4(4, 1) | BF4(3, 1);	// select SPI2

TP_RxDMACh->CPAR = (uint32_t)&TP_SPI->DR;

TP_RxDMACh->CMAR = (uint32_t)tp_rxbuf;

TP_RxDMACh->CNDTR = 3;

TP_RxDMACh->CCR = DMA_CCR_MSIZE16 | DMA_CCR_PSIZE16 | DMA_CCR_MINC | DMA_CCR_CIRC | DMA_CCR_EN;

}

// tx - DMA1 Channel 4 ch0

TP_TxDMACh->CCR = 0;

CLR_TPtx_FLAGS;

TP_TxDMACh->CPAR = (uint32_t)&TP_SPI->DR;

TP_TxDMACh->CMAR = (uint32_t)cmd;

TP_TxDMACh->CNDTR = 3;

TP_TxDMACh->CCR = DMA_CCR_MSIZE16 | DMA_CCR_PSIZE16 | DMA_CCR_MINC | DMA_CCR_DIR_M2P | DMA_CCR_EN;

}

Listing 2. Struktura danych i procedura odczytu stanu panelu wywoływana z przerwania timera

struct coords_

{

int16_t y, x;

};

struct tpd_

{

struct coords_ display;

volatile _Bool touched, released;

};

struct tpd_ tp;

void tp_wait4touch(void)

{

for (tp.touched = 0; !tp.touched;);

tp.released = 1;

}

struct tp_mtx_

{

int32_t An, Bn, Cn, Dn, En, Fn, Den;

};

static struct tp_mtx_ matrix;

#define TPDELTA 4

#define curr_tpad (*(struct coords_ *)(&tp_rxbuf[1]))

// tp sense, invoked from timer interrupt (50..100 Hz)

void tp_sense(void)

{

static struct coords_ prev_tpad;

static uint8_t tp_state = 0;

static _Bool tp_acqd = 0;

tp_state <<= 1;

if (TP_NOT_TOUCHED)

{

// not touched

if ((++ tp_state & 7) == 7)

{

tp.released = 1;

prev_tpad.x = 0;

prev_tpad.y = 0;

}

tp_acqd = 0;

}

else if (!tp.touched)

{

// was not touched, now touched

static _Bool acquire = 0;

acquire = 1;

if (tp_acqd)

{

int16_t dif;

if ((dif = curr_tpad.x - prev_tpad.x) < TPDELTA && dif > -TPDELTA

&& (dif = curr_tpad.y - prev_tpad.y) < TPDELTA && dif > -TPDELTA)

{

tp.display.x = (matrix.An * curr_tpad.x + matrix.Bn * curr_tpad.y + matrix.Cn) / matrix.Den;

if (tp.display.x < 0) tp.display.x = 0;

else if (tp.display.x > DISP_MAX_X) tp.display.x = DISP_MAX_X;

tp.display.y = (matrix.Dn * curr_tpad.x + matrix.En * curr_tpad.y + matrix.Fn) / matrix.Den;

if (tp.display.y < 0) tp.display.y = 0;

else if (tp.display.y > DISP_MAX_Y) tp.display.y = DISP_MAX_Y;

tp.touched = 1;

acquire = 0;

}

prev_tpad = curr_tpad;

tp_acqd = 0;

}

if (acquire)

{

tp_hw_start_acquire();

tp_acqd = 1;

}

}

}

Listing 3. Procedury kalibracji panelu

// compute callibration matrix based on data sampled from 3 points

static _Bool SetCalMatrix(const struct coords_ *CalCoords, struct coords_ *TPsample, struct tp_mtx_ *MtxPtr)

{

MtxPtr->Den = ((TPsample[0].x - TPsample[2].x) * (TPsample[1].y - TPsample[2].y))

-((TPsample[1].x - TPsample[2].x) * (TPsample[0].y - TPsample[2].y)) ;

if (MtxPtr->Den == 0) return 0;

MtxPtr->An = ((CalCoords[0].x - CalCoords[2].x) * (TPsample[1].y - TPsample[2].y)) -

((CalCoords[1].x - CalCoords[2].x) * (TPsample[0].y - TPsample[2].y)) ;

MtxPtr->Bn = ((TPsample[0].x - TPsample[2].x) * (CalCoords[1].x - CalCoords[2].x)) -

((CalCoords[0].x - CalCoords[2].x) * (TPsample[1].x - TPsample[2].x)) ;

MtxPtr->Cn = (TPsample[2].x * CalCoords[1].x - TPsample[1].x * CalCoords[2].x) * TPsample[0].y +

(TPsample[0].x * CalCoords[2].x - TPsample[2].x * CalCoords[0].x) * TPsample[1].y +

(TPsample[1].x * CalCoords[0].x - TPsample[0].x * CalCoords[1].x) * TPsample[2].y ;

MtxPtr->Dn = ((CalCoords[0].y - CalCoords[2].y) * (TPsample[1].y - TPsample[2].y)) -

((CalCoords[1].y - CalCoords[2].y) * (TPsample[0].y - TPsample[2].y)) ;

MtxPtr->En = ((TPsample[0].x - TPsample[2].x) * (CalCoords[1].y - CalCoords[2].y)) -

((CalCoords[0].y - CalCoords[2].y) * (TPsample[1].x - TPsample[2].x)) ;

MtxPtr->Fn = (TPsample[2].x * CalCoords[1].y - TPsample[1].x * CalCoords[2].y) * TPsample[0].y +

(TPsample[0].x * CalCoords[2].y - TPsample[2].x * CalCoords[0].y) * TPsample[1].y +

(TPsample[1].x * CalCoords[0].y - TPsample[0].x * CalCoords[1].y) * TPsample[2].y ;

return 1;

}

// wait until panel touched

void tp_wait4touch(void)

{

for (tp.touched = 0; !tp.touched;);

tp.released = 1;

}

// draw a cross for sampling point

static void DrawCross(uint16_t Xpos,uint16_t Ypos)

{

LCD_DrawLine(Xpos - 15, Ypos, Xpos - 2, Ypos, 0xffff);

LCD_DrawLine(Xpos + 2, Ypos, Xpos + 15, Ypos, 0xffff);

LCD_DrawLine(Xpos, Ypos - 15, Xpos, Ypos - 2, 0xffff);

LCD_DrawLine(Xpos, Ypos + 2, Xpos, Ypos + 15, 0xffff);

}

#define CM_OFF 45

static const struct coords_ CalPoints[3] = {

{CM_OFF, CM_OFF},

{CM_OFF, DISP_MAX_X - CM_OFF},

{DISP_MAX_Y - CM_OFF, DISP_MAX_X / 2}

};

void tp_calib(void)

{

struct coords_ TPsamp[3];

do

{

for (uint32_t i = 0; i < 3; i ++)

{

LCD_Clear(Black);

LCD_Text(MAX_Y / 2, CH_W, "Touch crosshair to calibrate", 0xffff, Black);

DrawCross(CalPoints[i].x, CalPoints[i].y);

tp_wait4touch();

TPsamp[i] = curr_tpad;

for (tp.released = 0; !tp.released;);

}

}

while (!SetCalMatrix(CalPoints, TPsamp, &matrix));

LCD_Clear(Black);

}