Listing 1. Konfiguracja interfejsu SPI dla PmodOLED

pmodOledSpi.Instance = SPI2;

pmodOledSpi.Init.Mode = SPI_MODE_MASTER;

pmodOledSpi.Init.Direction = SPI_DIRECTION_2LINES;

pmodOledSpi.Init.DataSize = SPI_DATASIZE_8BIT;

pmodOledSpi.Init.CLKPolarity = SPI_POLARITY_HIGH;

pmodOledSpi.Init.CLKPhase = SPI_PHASE_2EDGE;

pmodOledSpi.Init.NSS = SPI_NSS_SOFT;

pmodOledSpi.Init.BaudRatePrescaler = SPI_BAUDRATEPRESCALER_128;

pmodOledSpi.Init.FirstBit = SPI_FIRSTBIT_MSB;

pmodOledSpi.Init.TIMode = SPI_TIMODE_DISABLE;

pmodOledSpi.Init.CRCCalculation = SPI_CRCCALCULATION_DISABLE;

pmodOledSpi.Init.NSSPMode = SPI_NSS_PULSE_DISABLE;

HAL_SPI_Init(&pmodOledSpi);

Listing 2. Konfiguracja interfejsu UART dla PmodGPS

void PmodGPS_Config(void)

{

// Enable clock for GPIOD port required for 3DF and 1PPS pins.

__HAL_RCC_GPIOD_CLK_ENABLE();

// Configure the interrupts on PD10 (1PPS) and PD6 (3DF). The interrupts are active on both edges

// to indicate the connection and disconnection events.

GPIO_InitTypeDef GPIO_InitStruct;

GPIO_InitStruct.Mode = GPIO_MODE_IT_RISING_FALLING;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_VERY_HIGH;

GPIO_InitStruct.Pin = GPIO_PIN_6 | GPIO_PIN_10;

HAL_GPIO_Init(GPIOD, &GPIO_InitStruct);

HAL_NVIC_SetPriority(EXTI9_5_IRQn, 2, 0);

HAL_NVIC_SetPriority(EXTI15_10_IRQn, 2, 1);

HAL_NVIC_EnableIRQ(EXTI9_5_IRQn);

HAL_NVIC_EnableIRQ(EXTI15_10_IRQn);

pmodGpsUart.Instance = USART3;

pmodGpsUart.Init.BaudRate = 9600;

pmodGpsUart.Init.WordLength = UART_WORDLENGTH_8B;

pmodGpsUart.Init.StopBits = UART_STOPBITS_1;

pmodGpsUart.Init.Parity = UART_PARITY_NONE;

pmodGpsUart.Init.Mode = UART_MODE_TX_RX;

pmodGpsUart.Init.HwFlowCtl = UART_HWCONTROL_NONE;

pmodGpsUart.Init.OverSampling = UART_OVERSAMPLING_16;

pmodGpsUart.Init.OneBitSampling = UART_ONE_BIT_SAMPLE_DISABLE;

HAL_UART_Init(&pmodGpsUart);

}

Listing 3. Konfiguracja GPIO dla interfejsu UART

void PmodGPS_HAL_UART_MspInit(UART_HandleTypeDef *huart)

{

// Enable the clocks for GPIO pins used by the UART3 port (PC4, PC5).

__HAL_RCC_USART3_CLK_ENABLE();

__HAL_RCC_GPIOC_CLK_ENABLE();

GPIO_InitTypeDef GPIO_InitStruct;

GPIO_InitStruct.Mode = GPIO_MODE_AF_PP;

GPIO_InitStruct.Pull = GPIO_PULLDOWN;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_VERY_HIGH;

GPIO_InitStruct.Alternate = GPIO_AF7_USART3;

GPIO_InitStruct.Pin = GPIO_PIN_4 | GPIO_PIN_5;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

// Enable the interrupts generated by USART3 port.

HAL_NVIC_SetPriority(USART3_IRQn, 1, 0);

HAL_NVIC_EnableIRQ(USART3_IRQn);

}

Listing 4. Zmienne używane do odbioru danych z modułu PmodGPS

#define DATA_BUFFERS_COUNT 2

typedef struct

{

uint8_t buffer[128];

uint32_t index;

} DataBuffer;

DataBuffer dataBuffers[DATA_BUFFERS_COUNT];

uint32_t currentDataBuffer = 0;

uint8_t dataReady = 0;

Listing 5. Funkcja main przykładu

int main(void)

{

HAL_Init();

SystemClock_Config();

Led_Config();

Serial_Config();

for (uint32_t i = 0; i < DATA_BUFFERS_COUNT; i++) dataBuffers[i].index = 0;

PmodGPS_Config();

PmodGPS_Write("$PMTK605*31\r\n", 13);

PmodGPS_Read(dataBuffers[0].buffer, 1);

while(1)

{

while(dataReady == 0)

__NOP();

dataReady = 0;

// The buffer before the current one contains the data received from PmodGPS.

uint32_t readyDataBuffer =

(currentDataBuffer > 0) ? (currentDataBuffer - 1) : (DATA_BUFFERS_COUNT - 1);

// Write the data from the buffer to the serial port and clear the buffer.

Serial_Write((char*)dataBuffers[readyDataBuffer].buffer, dataBuffers[readyDataBuffer].index + 1);

dataBuffers[readyDataBuffer].index = 0;

}

}

Listing 6. Obsługa przerwania interfejsu UART

void HAL_UART_RxCpltCallback(UART_HandleTypeDef *huart)

{

if(dataBuffers[currentDataBuffer].buffer[dataBuffers[currentDataBuffer].index] == '\n')

{

dataReady = 1;

currentDataBuffer++;

if(currentDataBuffer == DATA_BUFFERS_COUNT) currentDataBuffer = 0;

} else {

dataBuffers[currentDataBuffer].index++;

}

PmodGPS_Read(&dataBuffers[currentDataBuffer].buffer[dataBuffers[currentDataBuffer].index], 1);

}

Listing 7. Fragmenty funkcji obsługujących diody Pmod8LD

void Pmod8LD_SetLed(Pmod8LD_Led led)

{

switch(led)

{

case Pmod8LD_Led0:

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_10, GPIO_PIN_SET);

break;

void Pmod8LD_ResetLed(Pmod8LD_Led led)

{

switch(led)

{

case Pmod8LD_Led0:

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_10, GPIO_PIN_RESET);

break;